Floorplan think in the logic first:

Data eneter series and out parallel with width of 8 bits

First module build :

Serializer:

Serializer do not generate start bit he is only start when fsm send it to him

Least sgnificant bit is first one serialized

Counter to count number ob bins

Sample the data only when V_input high

Busy is internal signal to determine if serializer work or not (I think it is not necessaey and should be included in FSM not serilizaer)

Positive clock edge clk and negative reset(Asynchronous Active-Low Reset)

We use parameter to be able to change it in the future

If it is valid input that is meaning we can take the data and start the serlizar process

If shift enable equal one then you should start shift the data to complete ther serilization process

Three cases tested to make sure that serilizer works good :

Rst=0 to see all data reset

Input data with EN=1 and  Valid=1 so data path correct

Input data one time when enable equal zero and one time with valid equal zero and we see that no data path that confirm that the system is working correct.

Second module:

Parity:

Parity is only module helps to know that there is an error in sending the data or not

I will use this test cases

Those three cases and case with the enable=0

FSM:

FSM iwill control the enable for the parity and the enable for the serilizerit it will determine which one will work and when.

Moore as it willdepend on the current state do not care about the input

TX_OUTPUT is high in the IDLE case (No transmission).

S0

IDLE

TX_OUTPUT=1

Busy=0

S4

Stop

Tx_output=1

Busy=1

MM

S1

Start

Tx_output=0

Busy=1

S2

Data

Tx_output=input data

Busy=1

S3

Parity

Tx_output=parity bit

Busy=1

S0=000

S1=001

S2=010

S3=011

S4=100

To move  from S0 to S1

We should load vinput

Data valid=1

From s1 to s2 the Ck must be 1 cycle

From s2 to s3 we should wait

#error in your code you should have count bit or wait 8 cycles to make sure all data are transferred

Or you can build in Fsm code

From s3 to s4:

To go to it we should first make that parity bit is enable if enable include it if not do not include it

If yes then include the parity bit include it in this bit

Signals inputs in the FSM:

V_INPUT check the data is valid or not

P_INPUT the parallel data itself

Clk

RST

Parity bit

Parity enable

Output signals

Busy

Mux selection

And the serializar enable

For mux he should choose between those:

signal IDLE to check if you are in IDLE or not

Signal start equal one when we are in start case

Signal data=1 when serializer is work

Signal parity=1 when we are

Signal stop=1 when we are passing the stop bit

We need also

State

Next state

Bit_count to count the data

And control signals for the

Mux and serializer and parity bit

Mux module:

Willbe 4 to 1 and out the selection bit for it

Start

Stop

Parity

Or data

00 start

01 data

10parity

11stop

4 cases every case will change the mux out and they should happens after each other

If the V_input equal one that is means that we should out the start data

mux=00

busy=1

Case 2 if the enable of serilizar is 1 that is mean we are in s2 place then

Mux=01

Stop till count equal 7

Case 3 if the parity enable is 1

Then mux=11

busy=1

Case 4 if the sop is one that is meaning

mux=10

busy=1

Fsm testing case:

We are going to check this cases:

Test cases we will apply on the FSM:

Reset

Busy 0 and Busy 1

Problem faced

All failed approximately

Why this happens

Notes i asked the Ai because iam tried to find the errors but i failed

In this line:

if (state == S0)

BUSY <= 0;

else

BUSY <= 1;

The main problems were related to timing: BUSY was registered inside the clocked block, so it used the old state and became delayed by one clock cycle, while SHT_EN and mux_sel were combinational and changed immediately with the current state. Also, the Testbench used extra #10 delays, causing it to check PARITY and STOP after the FSM had already moved to the next state. The solution is to make BUSY combinational with the other outputs and carefully place the Testbench delays so each state is checked during its correct clock cycle. Also, P_EN must be set before starting the transmission.

I have an error delay in clk pulse

Top modules mind map:

Also i enter all the inputs and outputs for the hole UART system

Only the signals and call the modules with the signals and call them and assign the value of start and stop

UART testbench:

What cases we need to test:

RSt

My code idea

Try to send all data 00000000

Try to send all data 11111111

Send two normal bits 010101110

Try to send parity bit 0 even

Try to send parity bit 1 odd

Try to send parity enable 0

Try to send parity enable 1

Test the parity bits use this two cases

Reset mid-transmission (top level)

Test the frame in every case

Letssssssssssss go

First case first stop failed why

I missed one clk cycle

Test case: Test UART transmission with P_INPUT = 00000000 and parity enabled, with P_BIT = 0.

Test case: Test UART transmission with P_INPUT = 00000000 and parity enabled, with P_BIT = 1.

Try parity enable equal 0 with the same case

Try to change parity bit to be equal 0 to conflict with stop but still pass

Test case: Test UART transmission with P_INPUT = 11111111 and parity enabled, with P_BIT = 0.

Test case: Test UART transmission with P_INPUT = 11111111 and parity enabled, with P_BIT = 1.

Test Case: 01011011 with Parity Enabled

Sending two different frames:

Sending two frames first fram all ones and second fram all zeros

draft

After polishing